************         Welcome to K&K International        ******************

P522

Before you starting the program, you must check your config.sys.

FILES=30

BUFFERS=30

The program runs from A:

To start the program enter P522DMO then return.

If you want to run the program from the harddisk, copy all files from a:

to harddisk.

To start program, enter P522DMO then return.

Enter global setup by type "3", then change the path setup to your disk.

The demo software generates the file A4STAT.DBF or A4STAT.TXT for statistic

information. In this file the results are appended for every test in runmode.

Remove the file to avoid the file filling the disk.

If "STORE" are set active, the data file A4M_STAT.DAT is generated.

The information is appended for every test in runmode.

Remove the file to avoid the file filling the disk.

The format of the A4STAT.DBF & A4STAT.TXT (as the DBF without no dbf header)

is:

"TYPE"      type number

"CHAIN"     chain type number

"DATE"      actual test date + month + year

"HOUR"      actual test hour

"MINUTE"    actual test minute

"USER_ID"   actual user id

"CH_A"      test result ch a            0=ok, 1=fail,   *=not active

"CH_B"      test result ch b            0=ok, 1=fail,   *=not active

"CH_C"      test result ch c            0=ok, 1=fail,   *=not active

"CH_D"      test result ch d            0=ok, 1=fail,   *=not active

"POL"       polarity test               0=ok, 1=fail,   *=not active

"SEN"       sensitivity test            0=ok, 1=fail,   *=not active

"RES"       resonance freq test         0=ok, 1=fail,   *=not active

"F"         F test (fres/Q)             0=ok, 1=fail,   *=not active

"LOUD"      loudness test               0=ok, 1=fail,   *=not active

"Q_TEST"    Q test 						0=ok, 1=fail,   *=not active

"2_LIM"     2-lim test                  0=ok, 1=fail,   *=not active

"3_LIM"     3-lim test                  0=ok, 1=fail,   *=not active

"NUMBER"    number code (optional)

"USER"      User result (optional)      0=ok, 1=fail,   *=not active

"U_CODE"    User code (optional)

"ZF_VAL"    resonance freq              value Hz,       *=not active

"ZV_VAL"    resonance freq value        value Ohm/dBR,  *=not active

"ZF_MIN"    minimum impendance freq     value Hz,       *=not active

"ZV_MIN"    mimimum impendance value    value Ohm/dBR,  *=not active

"ZQ_VAL"    Total Q of resonance        value,          *=not active

"F_VAL"     F = fres/Q                  value,          *=not active

"LOUDNESS"  loudness result             value,          *=not active

The format of the A4M_STAT.DAT version 2.2 is:

Every record is stored as:

Version header + Text header + User header +

Userdata + Record header + Active data.

Version header = Format code         2  byte bin

Record Length       2  byte bin

Text header =    Type number        12  byte char

Month               2  byte char

Date                2  byte char

Year                2  byte char

Hour                2  byte char

Minute              2  byte char

Secound             2  byte char

User header =    User fields         2  (0 = no user data)  byte bin

User record length  2  byte bin

User data =      User data length    2  byte bin

User data           1*(User data length)   byte bin

Record header =  Start freqency      2  byte bin

End frequency       2  byte bin

Test points         2  byte bin

Active channels     2  byte bin

Active data =    Channel no          2  byte bin

Scale log / lin     2  byte bin

Scale value (gain)  2  byte bin

Smoothing           2  byte bin

Data                2*(test points)  byte bin

Data - no smooth.   2*(test points)  byte bin

(only if smoothing > 0)

K&K International ApS

Sydvestvej 125 B

DK-2600 Glostrup

Denmark

Att. Ib Rasmussen

Telefone +45 44682294

+45 43422294

Fax      +45 43422494

e-mail   kk@kk-int.dk

www      kk-int.dk

kk-int.com